EE 6313

Advanced Microprocessor Systems

Project on

Design of 32-bit RISC microprocessor

Fall 2019

Done By

Kumar Ananda Chaya					     1001671413

Surabhi Chythanya Kumar                                       1001678544

Chandrashekar Mohan                                              1001657329

CONTENTS

| TOPICS | PAGE NUMBERS |
| --- | --- |
| INTRODUCTION | 3 |
| ALU INSTRUCTION SET | 4 |
| LD/ST INSTRUCTION SET | 6 |
| BRANCH INSTRUCTION | 9 |
| PIPELINE | 15 |
| INTERFACING | 26 |
| HAZARDS | 36 |
| EXTERNAL INTERRUPT HANDLING | 41 |

INTRODUCTION:

There are two types of microprocessor architecture namely, Reduced instruction set computer (RISC) architecture and complex instruction set computer (CISC) architecture.

As name indicates, instruction decoding in the CISC architecture will be complex and is mostly used for Application PC, laptop applications whereas there are relatively few instructions set in case of RISC architecture and is mostly used in smart phone processor architecture.

Our focus is to design a RISC architecture-based microprocessor.

PROJECT OVERVIEW:

The goal of this project is to design a 32-bit RISC microprocessor with load-store architecture with a 4-stage pipeline and Harvard architecture. The project also includes the instruction and data memory interfaces, register interface, and the entire pipeline control logic including full resolution of all structural, control, and data hazards.

The memory interface will only support a single asynchronous memory interface.

SPECIFICATIONS:

- Support 32-bit address bus, data bus and registers.

- All instructions in one fetch are 32-bit instructions.

- All instructions are aligned in 4N address.

- Harvard architecture constrained to one memory bus without cache.

- Supports 4 stages of pipeline.

- Thirty-two 32-bit registers are supported

- The memory space is provided with a single interface with A31..A2+BE3..0 addressing.

- The processor is of little-endian byte order.

- SP pointer convention that we have chosen as Pre decrement for PUSH and Post increment for POP

ALU INSTRUCTION SET

12

0

11

16

17

21

22

26

27

31

| OPCODE | DEST(RA) | SRC B(RB) | SRC C(RC) | K12 |
| --- | --- | --- | --- | --- |

- OPCODE is 5bits length(Bit 31-27).

- DEST(RA) is the destination register, SRC B(RB) and SRC C(RC) are the source registers which are of 5 bit length(Bit  26-12).

- K12 is an offset of 12 bits(Bit 11-0).

- All the registers are 32 bits.

- ALU operations are only between the registers.

OPERATION:

- MOV

Ex: MOV RA, RB – Moves contents of Reg B to Reg A

Here, RC=31 (escape code) and K12=0

- ADD

Ex: ADD RA, RB, RC – Adds the contents of Reg B and C and the result is stored in Reg A

Here K12=0.

- SUB

Ex: SUB RA, RB, RC – Adds the contents of Reg B and C and the result is stored in Reg A

Here K12=0.

- NEG

Ex: NEG RA, RB - Negates the contents of Reg B and stores the result is Reg A

- AND

Ex: AND RA, RB, RC – Performs AND operation of contents of Reg B and Reg C and stores the result in Reg A

- OR

Ex: OR RA, RB, RC – Performs OR operation of contents of Reg B and Reg C and stores the result in Reg A

- XOR

Ex: XOR RA, RB, RC – Performs XOR operation of contents of Reg B and Reg C and stores the result in Reg A

- NOT

Ex: NOT RA, RB – Performs 1’s compliment operation of the contents of Reg B and stores in Reg A.

- ROL

Ex: ROL RA, RB, RC – Performs circular left shift operation of contents of Reg B and contents of Reg C tells the number of times the bits of Reg B to be left shifted and stores the result in Reg A

- ROR

Ex: ROR RA, RB, RC – Performs circular right shift operation of contents of Reg B and contents of Reg C tells the number of times the bits of Reg B to be right shifted and stores the result in Reg A.

- ASR

Ex: ASR RA, #4, RB - Performs arithmetic right shift operation (used for signed number) of contents of Reg B i.e., #4  and contents of Reg C(#4) tells the number of times the bits of Reg B to be right shifted and stores the result in Reg A.

- LSR

Ex: LSR RA, #2, RB - Performs logical right shift operation (used for unsigned number) of contents of Reg B i.e., #2 and contents of Reg C(#2) tells the number of times the bits of Reg B to be right shifted and stores the result in Reg A.

- ASL/SHL

Ex: LSR RA, #3, RB – Performs arithmetic/logical right left operation of contents of Reg B and contents of Reg C(#2) tells the number of times the bits of Reg B to be right shifted and stores the result in Reg A.

- MUL

Ex: MUL RA, RB, RC – Multiplies the contents of Reg B and Reg C and stores the result in Reg A

- NOP

It has no operation and it increments PC by 4.

0

31

LD/ST INSTRUCTION SET

9

10

11

12

16

17

21

22

26

27

| OPCODE | DEST(RA) | SRC B(RB) | SRC C(RC) | OPT | K10 |
| --- | --- | --- | --- | --- | --- |

- OPCODE is 5bits length (Bit 31-27).

- DEST(RA) is the destination register, SRC B(RB) and SRC C(RC) are the source registers which are of 5bit length (Bit 26-12).

- Option field (OPT) is a 2bit field (Bit 11-10).

- K10 is a signed offset field of 10 bits (Bit 9-0).

- All the registers are 32 bits.

- Memory operations are done by LD/ST.

OPT Field

| 0 | NOP | No operation |
| --- | --- | --- |
| 1 | PUSH/POP | Reg B +/-4 |
| 2 | DEC | Reg C -1 |
| 3 | INC | Reg C +1 |

LD/ST operation

For LD, RA,[RB+2N*RC+K10]

For ST, [RB+2N*RC+K10], RA

This is indirect, indexed, offset addressing mode.

Consider C code

int16_t  y[100];

i=100;

y[--i]=0;

Equivalent Assembly code is:

MOV RA, #0

MOV RB, &y[100]   //RB is the base register

MOV RC, i           //RC is decrement register (according to our convention)

ST [RB+2*RC], RA

0

31

MAPPING OF ABOVE INSTRUCTION

9

10

11

12

16

17

21

22

26

27

| OPCODE | SRC(RA) | SRC B(RB) | SRC C(RC) | OPT | K10 |
| --- | --- | --- | --- | --- | --- |

0

31

9

10

11

12

16

17

21

22

26

27

| 10101 | #0 | &y[100] | i | 10 | 0000000000 |
| --- | --- | --- | --- | --- | --- |

The above example is indirect indexed addressing mode. Here offset is zero

Consider C code

int16_t  y[100];

Equivalent Assembly code is:

MOV RB, &y[0]   //RB is the base register

MOV RC, i           //RC is increment register (according to our convention)

LD RA, [RB+2*RC]

0

31

MAPPING OF ABOVE INSTRUCTION

9

10

11

12

16

17

21

22

26

27

| OPCODE | DEST(RA) | SRC B(RB) | SRC C(RC) | OPT | K10 |
| --- | --- | --- | --- | --- | --- |

0

31

9

10

11

12

16

17

21

22

26

27

| 10000 | RA | &y[0] | i | 11 | 0000000000 |
| --- | --- | --- | --- | --- | --- |

The above example is indirect indexed addressing mode. Here offset is zero

- LD32

Ex: LD32 RA,[RB+4*RC+#12]- Loads 32 bit value from memory to reg A

- LDU16

Ex: LDU16 RA,[RB+2*RC+#12]- Loads unsigned 16 bit value from memory to     reg A

- LDS16

Ex: LDS16 RA,[RB+2*RC+#12]- Loads signed 16 bit value from memory to reg A

- LDU8

Ex: LDU8 RA,[RB+2*RC+#12]- Loads unsigned 8 bit value from memory to reg A

- LDS8

Ex: LDS8 RA,[RB+2*RC+#12]- Loads signed 8 bit value from memory to reg A

0

31

MAPPING OF LD32 INSTRUCTION

9

10

11

12

16

17

21

22

26

27

| OPCODE | DEST(RA) | SRC B(RB) | SRC C(RC) | OPT | K10 |
| --- | --- | --- | --- | --- | --- |

0

31

9

10

11

12

16

17

21

22

26

27

| 10000 | RA | RB | RC | 00 | 0000001100 |
| --- | --- | --- | --- | --- | --- |

LD32 RA,[RB+4*RC+#12]- Loads 32 bit value from memory to reg A

- Here OPCODE is (16)10

- OPT field is 00 which indicates there’s no special operation.

- From the above example, K10 offset field is (12)10

- ST32

Ex: ST32 [RB+4*RC+K10], RA Stores 32 bit value from Reg A to memory

- ST16

Ex: ST16 [RB+2*RC+K10], RA Stores 16 bit value from reg A to memory

- ST8

Ex: ST8 [RB+RC+K10], RA Stores 8 bit value from reg A to memory

0

31

MAPPING OF ST32 INSTRUCTION

9

10

11

12

16

17

21

22

26

27

| OPCODE | SRC A(RA) | SRC B(RB) | SRC C(RC) | OPT | K10 |
| --- | --- | --- | --- | --- | --- |

0

31

9

10

11

12

16

17

21

22

26

27

| 10101 | RA | RB | RC | 00 | 0000001100 |
| --- | --- | --- | --- | --- | --- |

ST32 [RB+4*RC+#12], RA Stores 32 bit value from Reg A to memory

- Here OPCODE is (21)10

- OPT field is 00 which indicates there’s no special operation.

- From the above example, K10 offset field is (12)10

0

31

MAPPING OF PUSH INSTRUCTION

9

10

11

12

16

17

21

22

26

27

| OPCODE | SRC A(RA) | SRC B(RB) | SRC C(RC) | OPT | K10 |
| --- | --- | --- | --- | --- | --- |

0

31

9

10

11

12

16

17

21

22

26

27

| 10101 | RA | 11100 | 11111 | 01 | 0000000100(+4) |
| --- | --- | --- | --- | --- | --- |

PUSH RA -> The SP is Pre decremented by 4 bytes and stores the 32-bit value from Reg A to stack memory

- PUSH operation is like store operation. Hence, OPCODE is (21)10

- RB is the stack pointer registers

- RC is 31 which is escape value since it is not used.

- OPT field is 01 which indicates the PUSH.

- From the above example, K10 offset field is +4.

0

31

MAPPING OF POP INSTRUCTION

9

10

11

12

16

17

21

22

26

27

| OPCODE | SRC A(RA) | SRC B(RB) | SRC C(RC) | OPT | K10 |
| --- | --- | --- | --- | --- | --- |

0

31

9

10

11

12

16

17

21

22

26

27

| 10000 | RA | 11100 | 11111 | 01 | 0000000000 |
| --- | --- | --- | --- | --- | --- |

POP RA -> The SP is Post incremented by 4 bytes and stores the 32-bit value from stack memory to Reg A.

- POP operation is like load operation. Hence, OPCODE is (16)10

- RB is the stack pointer registers

- RC is 31 which is escape value since it is not used.

- OPT field is 01 which indicates the POP.

- From the above example, K10 offset field is 0.

BRANCH INSTRUCTIONS

OPERATION:

- GOTO {LABEL}

Ex: GOTO #0x20000000.

| 1000 0000 | GOTO |
| --- | --- |
| 1000 0004 | 20000000 |

Here PC will be loaded with the absolute 32-bit value.

From the above example, OPCODE GOTO will occupy 5 bits so the 32-bit value won’t fit in the remaining 27 bits. Hence, the absolute 32-bit value is stored in next memory location (next instruction).

PC←abs32

The above operation is equivalent to LD32 PC,[PC]

0

31

MAPPING OF GOTO INSTRUCTION

9

10

11

12

16

17

21

22

26

27

| OPCODE | DEST(RA) | SRC B(RB) | SRC C(RC) | OPT | K10 |
| --- | --- | --- | --- | --- | --- |

0

31

9

10

11

12

16

17

21

22

26

27

| 11000 | 11110 | 11110 | 11111 | 00 | 0000000000 |
| --- | --- | --- | --- | --- | --- |

GOTO #0x20000000- Loads 32 bit value from memory to reg A

- Here GOTO is like load operation (LD32 PC,[PC]). Hence, OPCODE is (16)10

- RA and RB are PC registers whose number is 30.

- OPT field is 00 which indicates there’s no special operation.

- From the above example, K10 offset field is (12)10

- CALL {LABEL}

Ex: CALL #0x20000000.

| 1000 0000 | CALL |
| --- | --- |
| 1000 0004 | 20000000 |
| 1000 0008 | “Returns here” |

Here Initially, value of PC will be saved in PCT i.e., temporary PC and then PC will be loaded with the absolute 32-bit value.

The PC is incremented by 4bytes and it stores the contents to PCT (save PC). Then PC gets the next instruction that is the absolute 32-bit value.

PCT←PC+4

PC←abs32

To load PCT, we must add a custom logic.

The above operation is equivalent to LD32 PC,[PC]

0

31

MAPPING OF CALL INSTRUCTION

9

10

11

12

16

17

21

22

26

27

| OPCODE | DEST(RA) | SRC B(RB) | SRC C(RC) | OPT | K10 |
| --- | --- | --- | --- | --- | --- |

0

31

9

10

11

12

16

17

21

22

26

27

| 11001 | 11110 | 11110 | 11111 | 00 | 0000000000 |
| --- | --- | --- | --- | --- | --- |

CALL #0x20000000- Loads 32 bit value from memory to reg A

- Here CALL is like load operation (LD32 PC,[PC]). Hence, OPCODE is (16)10

- RA and RB are PC registers whose number is 30.

- OPT field is 00 which indicates there’s no special operation.

- From the above example, K10 offset field will be zero

- RETURN

Here PC gets PCT

PC←PCT

To execute this instruction, we need to add custom logic.

0

31

MAPPING OF RETURN INSTRUCTION

11

12

16

17

21

22

26

27

| OPCODE | DEST(RA) | SRC B(RB) | SRC C(RC) | K12 |
| --- | --- | --- | --- | --- |

0

31

11

12

16

17

21

22

26

27

| 11010 | ? | ? | ? | ? |
| --- | --- | --- | --- | --- |

- INT N

Ex: INT #12.

Here Initially, value of PC will be saved in PCI then FLAGS will be pushed to FLAGSI. Then PC will be loaded with respective interrupt vector address.

PCI[N]←PC

FLAGSI[N]←FLAGS

PC←ivt[N]

To load PCI and FLAGSI, we need to add a custom logic.

The above operation is equivalent to LD32 PC,[BASE+N*4]

0

31

MAPPING OF INT INSTRUCTION

9

10

11

12

16

17

21

22

26

27

| OPCODE | DEST(RA) | SRC B(RB) | SRC C(RC) | OPT | K10 |
| --- | --- | --- | --- | --- | --- |

0

31

9

10

11

12

16

17

21

22

26

27

| 11011 | 11110 | 11010 | 11111 | 00 | 0000001100 |
| --- | --- | --- | --- | --- | --- |

INT #12

- Hence, OPCODE is (16)10  (LD32 PC,[BASE+N*4]).

- RA is PC (30).

- RB is base register (26).

- RC is escape value 31.

- OPT field is 00 which indicates there’s no special operation.

- From the above example, K10 offset field will be #12.

- PCI[N] and FLAGSI[N] are fast registers and are only in WB stage of pipeline. These can be written into WB and read over WB and there are no extra pipeline signals

- No access to PCI and FLAGSI

- RETI

Here Initially, flags will be Popped and the PC is PC will be restored by PCI.

FLAGS←FLAGSI[N]

PC←PCI[N]

To load PC and FLAGS, we need to add a custom logic.

The above operation is equivalent to LD32 PC,[BASE+N*4](?)

0

31

MAPPING OF RETI INSTRUCTION

9

10

11

12

16

17

21

22

26

27

| OPCODE | DEST(RA) | SRC B(RB) | SRC C(RC) | OPT | K10 |
| --- | --- | --- | --- | --- | --- |

0

31

9

10

11

12

16

17

21

22

26

27

| 11100 | ? | ? | ? | ? | ? |
| --- | --- | --- | --- | --- | --- |

INT #12

- Hence, OPCODE is (16)10  (LD32 PC,[BASE+N*4]).

- RA is PC (30).

- RB is base register (26).

- RC is escape value 31.

- OPT field is 00 which indicates there’s no special operation.

- From the above example, K10 offset field will be #12.

- BRA rel27

Ex: BRA #8.

| BRA | #8 |
| --- | --- |

Here PC will be loaded with the PC and relative offset.

PC←PC + rel27*4

We need to add custom logic to the above operation

0

31

MAPPING OF BRA INSTRUCTION

26

27

| OPCODE | OFS27 |
| --- | --- |

0

31

27

| 11101 | 000 0000 0000 0000 0000 0000 1000 |
| --- | --- |

The branch instruction can jump in the range of +/-228 or +/- 256MB jump

- B cond rel23

Ex: BRA Z #24.

| BRA | cond | #24 |
| --- | --- | --- |

When if condition is true PC will be loaded with the PC and relative offset.

If(cond),PC←PC + rel27*4

Custom logic is needed added for the above operation

0

31

MAPPING OF BRA Z INSTRUCTION

22

23

26

27

| OPCODE | COND | OFS23 |
| --- | --- | --- |

0

31

22

23

26

27

| 11110 | cond | 000 0000 0000 0000 0001 1000 |
| --- | --- | --- |

The conditional branch instruction can jump in the range of +/-224 or +/- 16MB jump

- RCALL rel27

Ex: RCALL #8.

| RCALL | #8 |
| --- | --- |

Here PC will be loaded with the PCT (save PC) and PC will be loaded with relative offset.

PCT←PC

PC←PC + rel27*4

Custom logic is needed for the above operation

0

31

MAPPING OF RCALL INSTRUCTION

26

27

| OPCODE | OFS27 |
| --- | --- |

0

31

27

| 11111 | 000 0000 0000 0000 0000 0000 1000 |
| --- | --- |

PIPELINE

There are 4 stages of pipeline.

- Instruction Fetch

- Read Register/Address Generation

- Execute/ Fetch Operands

Memory Interface

Register Interface

Write Back

DFF

DFF

DFF

Execute/ Fetch Operands

Read Register/ Address Generation

Instruction Fetch

Write Back

Flush

Stall

- INSTRUCTION FETCH (IF)

This is the 1st stage of pipeline. It fetches next instructions.

PC_INIT

XACT_IF_ADD

XACT_IF_DATA

0 Reset

1 Normal

2 Write PC/Flush

32

0

PC

+4

PC Fetch

- When reset, the PC Fetch is loaded with PC_INIT which is reset vector address. Zero  is the address of the first instruction to be executed. This can be written as: LD PC,[#0].

- During normal operation, PC Fetch will be incremented by 4.

- When write PC/Flush, the PC loads the new value to PC Fetch.

24

24

IF Operation:

PCF	0	4	8	12	16	24	28	32	36	40

PC	0	0	0	4	4	24	24	24	28	28

Branch

Flush

During normal operation, PC Fetch will be incrementing by 4. During branch instruction, PC will be loaded with new value, and then it writes to PC Fetch.

Signal generated from IF

OPCODE                       IR[31:27]

OPT                                IR[11:10]                 if(OPCODE = LD/ST|PUSH |POP)

K10                                 IR[9:0]                     if(OPCODE = LD/ST|PUSH|POP)

K12                                 IR[11:0]                   if(OPCODE = ALU)

COND                            IR[26:23]                  if(OPCODE = BRA COND)

N                                     IR[9:2]                      if(OPCODE = INT)

RD_REGA_EN = 1                                          if(OPCODE != RET|RETI|BRA|

BRA COND|ALU)

RD_REGA_NUM          IR[26:22]                  if(RD_REGA_EN)

RD_REGB_EN = 1                                          if(OPCODE != RETRETI|BRA|BRA COND)

RD_REGB_NUM          IR[21:17]                  if(RD_REGB_EN)

RD_REGC_EN = 1                                          if(OPCODE != RET|RETI|BRA|BRA COND)

RD_REGC_NUM          IR[16:12]                  if(RD_REGC_EN)

RD_MEM_EN = 1                                           if( OPCODE = POP|LD)

WR_MEM_EN = 1                                          if( OPCODE = PUSH|ST)

WR_REG_EN = 1                                           if( OPCODE! = POP|LD)

WR_REG_NUM                                             if( WR_REG_EN)

WR_REG_DATA                                           if( WR_REG_EN)

WR_INCDEC_REG_EN = 1                          if( OPCODE = LD|ST && OPT =2|OPT =3)

WR_INCDEC_REG_NUM                            if(WR_INCDEC_REG_EN)

WR_SP_EN = 1                                              if( OPCODE = PUSH|POP)

WR_PCT_EN = 1                                            if( OPCODE = CALL | RCALL)

WR_PC_EN=1                                                if( OPCODE = BRA|BRA COND|RETI|

INT|CALL|RESET)

WR_FLAG_EN = 1                                         if( OPCODE = RETI)

- READ REGISTER/ ADDRESS GENERATION (RR/ADGEN)

RR part of RR/ADGEN

RR is used to get ALU arguments.

- Signals to Register interface

RI_RD_REGA_EN = SRC A != 31

RI_RD_REGA_NUM = SRC A

RI_RD_REGB_EN = SRC B != 31

RI_RD_REGB_NUM = SRC B

RI_RD_REGC_EN = SRC C != 31

RI_RD_REGC_NUM = SRC C

- SRC A,B and C are the signals from IF stage.

- In this stage, the data dependency hazard can be detected.

- Signals from Register interface

RI_RD_REGA_DATA – This can be SRC A for ST or DEST for ALU

RI_RD_REGB_DATA – This can be SRC B for ADGEN or ARG1 for ALU

RI_RD_REGC_DATA – This can be SRC C for ADGEN or ARG2 for ALU

ARGUMENT GERATION TO EX STAGE

RI_RD_REGC_DATA

RI_RD_REGB_DATA

ARG1_DATA

(TO EX)

ARG2_DATA

(TO EX)

K12

K12

Fn(SRC C)

Fn(SRC B)

Signals generated from RR

RD_REGB_NUM|RR

ALU_ARG1

If(REGB_EN|RR&&opcode=ALU)

RD_REGC_NUM|RR

ALU_ARG2

If(REGC_EN|RR&&opcode=ALU)

RD_REGB_EN|RR

RD_REGB_NUM|RR&&opcode>15

RI_RD_REGB_DATA

RI_RD_REGC_DATA

RD_REGC_EN|RR

RD_REGC_NUM|RR&&opcode>15

RI_RD_REGA_DATA

RD_REGA_EN|RR

RD_REGA_NUM|RR&&opcode=ST|PUSH

If(opcode=POP|PUSH)

WR_SP_DATA = SP+K10

RD_REGB_EN

If(RD_REGB_NUM=27)

WR_PCT_DATA

If(CALL|RCALL)

If(opcode=GOTO|CALL)

WR_PC_DATA = abs32

WR_PC_DATA = PCT|PCI

If(opcode=RET|RETI)

WR_PC_DATA=PC_DATA  + OFFSET

If(BRA|BRACOND|RCALL)

WR_PC_DATA=IVT[N]

If(opcode = INT N)

WR_PC_DATA=reset vector address = 0

If(RESET)

ADGEN part of RR/ADGEN

ADGEN is used to generate memory address for LD/ST operation

RI_RD_REGB_DATA

ADDRESS

RW_MEM_ADD

SRC B

Σ

0

SRC B == 31

RI_RD_REGC_DATA

SRC C

<<

SRC C == 31

K10

0

N

[SRCB + SRCC<<2+SIGNED(K10)]

- Some of the signals are denoted different in further pipelines stages but they are of the same function.

Ex: RI_RD_REG_DATA in RR stage becomes WR_REG_DATA in FO stage.

OVERVIEW

| INPUTS | OUTPUTS | PASS THROUGH |
| --- | --- | --- |
| OPCODE | ALU_ARG1 | RD_MEM_EN |
| OPT | ALU_ARG2 | WR_MEM_EN |
| K10 | RI_RD_REGA_DATA | WR_REG_EN |
| K12 | RI_RD_REGB_DATA | WR_REG_NUM |
| N | RI_RD_REGC_DATA | WR_INCDEC_REG_EN |
| RD_REGA_NUM | WR_PC_DATA | WR_INCDEC_REG_NUM |
| RD_REGB_NUM | WR_PCT_DATA | WR_SP_EN |
| RD_REGC_NUM | WR_FLAG_DATA | WR_PCT_EN |
| RD_REGA_EN | RW_MEM_ADD | WR_PC_EN |
| RD_REGB_EN | WR_SP_DATA | WR_FLAG_EN |
| RD_REGC_EN |  |  |
| COND |  |  |

- FETCH OPERAND/EXECUTION (FO/EX)

32

FLAG_DATA

(TO WB)

ARG1_DATA

(FROM RR)

EX part of FO/EX

32

5

32

32

32

ALU DATA

DONE_EX

WR_REG_DATA

(TO WB)

Fn (OPCODE)

LD DATA

FROM FO

OPCODE

(FROM RR)

ARG2_DATA

(FROM RR)

- ALU gets the operands from the RR stage and perform ALU operations depending on the OPCODE.

- The MUX selects the ALU data or LD data depending on the OPCODE logic which generates the WR_REG_DATA.

FO part of FO/EX

MEMORY INTERFACE

LD_DATA

XACT_FO_ADD

XACT_FO_REQ

XACT_FO_DATA

XACT_FO_DONE

DONE_FO

RD_MEM_EN

RD_MEM_ADD

RR

D                    Q

The LOAD DATA is generated from memory interface and DONE_FO is obtained as soon as LOAD_DATA is given from memory.

Signals generated from FO/EX stage

WR_MEM_DATA

RI_RD_REGA_DATA

RI_RD_REGA_DATA will become WR_MEM_DATA as it will be written into memory in WB stage.

RD_MEM_EN

WR_REG_DATA

RW_MEM_ADD

WR_INCDEC_REG_DATA

RI_RD_REGC_DATA

RI_RD_REGC_DATA will become WR_INCDEC_REG_DATA as it will be written into register in WB stage.

WR_FLAG_DATA

flag output

Flag output will become WR_FLAG_DATA as it will be written into flag in WB stage

DONE SIGNAL GENERATION

DONE_EX

OP={ALU}

DONE_FO

RD_MEM_EN

DONE_FO

OVERVIEW

| INPUTS | OUTPUTS | PASS THROUGH |
| --- | --- | --- |
| RI_RD_REGA_DATA | WR_MEM_DATA | WR_PCT_DATA |
| RD_MEM_EN | WR_FLAG_DATA | WR_MEM_EN |
| ALU_ARG1 | WR_REG_DATA | WR_SP_EN |
| ALU_ARG2 | WR_INCDEC_DATA | WR_SP_DATA |
| RI_RD_REGC_DATA |  | WR_PCT_EN |
| RW_MEM_ADD |  | WR_PCT_DATA |
| OPCODE |  | WR_PC_EN |
| OPT |  | WR_PC_DATA |
| COND |  | WR_REG_EN |
| N |  | WR_REG_NUM |
|  |  | WR_INCEDEC_REG_EN |
|  |  | WR_INCEDEC_REG_NUM |
|  |  | WR_FLAG_EN |

- WRITE BACK (WB)

This is the last stage in the pipeline and it writes to registers and memory.

INPUTS TO WB STAGE OF PIPELINE

Write to registers signals

WR_REG_EN (1 b)

WR_REG_NUM (5 b)

WR_REG_DATA (32 b)

Write to memory signals

WR_MEM_EN (1 b)

WR_MEM_ADD (32 b)

WR_MEM_DATA (32 b)

Write to PC signals

WR_PC_EN (1 b)

WR_PC_DATA (32 b)

Write to flags signals

WR_FLAGS_EN (1 b)

WR_FLAGS_DATA (32 b)

Write to SP signals

WR_SP_EN (1 b)

WR_SP_DATA (32 b)

Increment and decrement registers

INC_DEC_REG_EN (1 b)

INC_DEC_REG_NUM (5 b)

INC_DEC_REG_DATA (32 b)

Write to PCT signals

WR_PCT_EN (1 b)

WR_PCT_DATA (32 b)

INTERFACE TO MEMORY

XACT_WB_REQ (1 b)

XACT_WB_ADD (32 b)

XACT_WB_DATA (32 b)

XACT_WB_DONE (1 b)

INTERFACE TO REGISTERS

RI_WR_REGA_EN (1 b)

RI_WR_REGA_NUM (5 b)

RI_WR_REGA_DATA (32 b)

RI_WR_REGB_EN (1 b)

RI_WR_REGB_NUM (5 b)

RI_WR_REGB_DATA (32 b)

RI_WR_SP_EN (1 b)

RI_WR_SP_DATA (32 b)

RI_WR_PC_EN (1 b)

RI_WR_PC_DATA (32 b)

RI_WR_PCT_EN (1 b)

RI_WR_PCT_DATA (32 b)

RI_WR_PCT_EN (1 b)

RI_WR_PCT_DATA (32 b)

FEEDBACK SIGNAL

DONE_WB (1 b)

FLUSH (1 b)

- In WB it the writes to registers and memory and then PC is incremented by 4.

- During branch, WB writes to PC and sends the FLUSH signal.

- After write operation, DONE_WB signal is generated.

- RI_{}_EN signal detects data hazards.

SIGNAL FLOW IN WB STAGE

Write to the register

WR_REG_NUM

WR_REG_EN

WR_REG_DATA

WR_MEM_EN

Write to the memory

RW_MEM_ADD

WR_MEM_DATA

WR_INCDEC_REG_EN

WR_INCDEC_REG_NUM

Write to inc/dec register

WR_INCDEC_REG_DATA

WR_FLAG_EN

Write to flag register

WR_REG_NUM=26

WR_FLAG_DATA

WR_SP_EN

WR_REG_NUM=28

Write to SP register

WR_SP_DATA

WR_PC_EN

Write to PC register

WR_REG_NUM=30

WR_PC_DATA

WR_PCT_EN

WR_PCT_DATA

WR_REG_NUM=27

Write to PCT register

- Explicit write has highest priority over normal functionality.

Suppose if we write to FLAGS, the normal operation result of FLAGS is overwritten with the write operation result.

WR_FLAG_EN

WR_FLAG_DATA

NOT(WR_REG_EN&WR_REG_NUM==FLAGS)

Suppose if we write to SP, the normal operation result of SP is overwritten with the write operation result.

Ex : POP SP

WR_SP_EN

WR_SP_DATA

NOT(WR_REG_EN&WR_REG_NUM==SP)

Suppose if we write to PC, the normal operation result of PC is overwritten with the write operation result.

Ex : BRA COND

WR_PC_EN

WR_PC_DATA

NOT(WR_REG_EN&WR_REG_NUM==PC)

INTERFACING

DESIGN OF EXTERNAL MEMORY INTERFACE

32

XACT_{ }_EN

ADD

ADD

32

DATA

DATA

XACT_{ }_ADD

XACT_{ }_DATA

RD

RD

XACT_{ }_DIR(R/W)

WR

WR

READY

XACT_{ }_DONE

- { } can be any of the 3 stages: IF, FO and WB.

- XACT_{ }_DIR indicates direction for read and write operation.

PRIORITY

HIGHER PRIORITY

IF

FO

WB

IF_EN

EN

PRIORITY ENCODER

FO_EN

EN

WB_EN

EN

- WB stage always has the highest priority.

- IF stage is always hardwired to zero.

ADDRESS SELECTION

XACT_IF_ADD

XACT_FO_ADD

ADD

XACT_WB_ADD

32

3

{  } _ EN

DIR GENERATION

DIR

XACT_WB_DIR

XACT_IF_DIR

XACT_FO_DIR

{  } _ EN

3

- The Address and DIR signals are selected by enable signals that are selected by priority encoder.

- For read operation (IF, FO), DIR=1 and for write operation (WB), DIR=0.

Truth table for priority encoder

| I2 | I1 | I0 | STAGE |
| --- | --- | --- | --- |
| X | X | 1 | WB |
| X | 1 | 0 | FO |
| 1 | 0 | 0 | IF |

GENERATION OF DONE SIGNAL

IF_EN

READY

IF_DONE

FO_EN

FO_DONE

READY

WB_EN

READY

WB_DONE

- Significance of DONE signal: As soon as any of the stage of the pipeline completes its operations, the respective stage sends a DONE signal in order to avoid overwriting.

- There is no DONE signal for RR stage of pipeline as it is only between registers and is fast.

DATA SELECTION

XACT_IF_DATA

DATA

XACT_FO_DATA

XACT_WB_DATA

- DIR signal indicates the direction of dataflow.

- To facilitate bidirectionality, transceiver is used.

- IF, FO and WB EN signals selects one of the stages according to the priority.

TIMING DIAGRAM

ADDRESS

DATA

READY

RD

DIR

WR

- Bus cycle runs 4 times faster than instruction cycle.

- Clock is stabilized by Phase Lock Loop.

- Read and write is based on DIR.

GENERATION OF MEMORY INTERFACE SIGNALS

IF STAGE

MEMORY

XACT_IF_REQ=1

XACT_IF_DONE

XACT_IF_DATA

XACT_IF_R/~W=1

XACT_IF_S/~U=0

XACT_IF_SIZE=3

XACT_IF_ADD

PC

IF_DATA

IF_DONE

FO STAGE

MEMORY

XACT_FO_SIZE

XACT_FO_REQ

XACT_FO_ADD

XACT_FO_S/~U

XACT_FO_R/~W=1

XACT_FO_DATA

XACT_FO_DONE

FO_DONE

RD_MEM_EN

RW_MEM_ADD

Fn(OPCODE)

Fn(OPCODE)

WR_REG_DATA

WB STAGE

MEMORY

XACT_WB_REQ

XACT_WB_S/~U

XACT_WB_SIZE

XACT_WB_R/~W=0

XACT_WB_DONE

XACT_WB_DATA

XACT_WB_ADD

WR_MEM_EN

RW_MEM_ADD

Fn(OPCODE)

Fn(OPCODE)

WB_DONE

WR_MEM_DATA

D R0 Q

R1

R2

R3

R4

R5

R6

R7

R8

R9

R10

R11

R12

R13

R14

R15

R16

R17

R18

R19

R20

R21

R22

R23

R24

R25

R26

R27

R28

R29

R30

32

WR_REG_DATA

REGISTER INTERFACE

32

REG_DATA

RI_{}_DATA

32

SPECIAL SIGNALS

32

RI_{}_NUM

5

OE=0

- Registers R0-R25 are general purpose registers.

- Registers R26-R30 are special registers.

| REGISTERS |  |
| --- | --- |
| 0-25 | General Purpose |
| 26 | BASE |
| 27 | PCT |
| 28 | SP |
| 29 | FLAGS |
| 30 | PC |
| 31 | escape code |

- WR_REG_DATA is normal write signal.

- REG_DATA is for increment/decrement operations.

OE=0

Output enable              is always on.

WR_REG_DATA                   NOTHING                    R26  BASE

REG_DATA                           WR_PCT                       R27  PCT

WR_FLAGS                  R28  FLAGS

WR_SP                          R29  SP

WR_PC                          R30 PC

MEMORY INTERFACE

Memory interface for ST operations

For example,

STB [R0], R1

- Let’s assume that XACT_ADD=12345679h i.e., R0=12345679h

- Since it is a byte operation, size=1. Opcode tells the size

- Data is LSB aligned

- Memory data is a function of DATA, SIZE and ADDRESS.

MEM_DATA= fn(DATA, SIZE, ADD1:0)

Where ADD1:0 are bank enable bits and upper 30 bits are address

| MEM_BE0 | 0 | 0 |
| --- | --- | --- |
| MEM_BE1 | 0 | 1 |
| MEM_BE2 | 1 | 0 |
| MEM_BE3 | 1 | 1 |

MEM_BE=fn(ADD1:0, SIZE)

BE

BE3    1

BE2    1

BE1    0

BE0    1

DATA7:0

XX

XX

REGISTER

MEMORY

XX

DATA23:16

DATA15:8

DATA7:0

DATA31:24

MEM_ADD=ADD31:2

- Since last 2 bits of R0 is 01(0x12345679), BE1 is selected.

- The rest of the bytes are ignored(xx).

All switch combinations

REGISTER

DATA31:24

DATA23:17

MEMORY

DATA7:0

DATA15:8

There are totally 9 possible combinations.

Memory interface for LD operations

For example,

LDSB R1, [ R0]

- Let’s assume that XACT_ADD=12345679h i.e., R0=12345679h

- Since it is a byte operation, size=1. Opcode tells the size

- Data is LSB aligned

- All registers are 32bits wide. Hence, it must be signed or unsigned extended.

- Signed or unsigned extension depends on OPCODE.

REGISTER

MEMORY

XX

XX

XX

DATA7:0

DATA15:8

DATA23:17

DATA31:24

DATA15:8

- A byte of data is loaded to lower 8 bits of register and the rest of the bits are sign extended for this example.

- It is a function of SIGNED/UNSIGNED, SIZE, ADDRESS1:0

All switch combinations

REGISTER

MEMORY

S

S

Z

Z

Z

4:1

5:1

2:1

2:1

5:1

(SSSSSSSS)

(00000000)

DATA15:8

DATA23:16

DATA31:24

fn(S/U, SIZE,ADD1:0)

- The lower 8 bits always gets data from 4:1 mux connected from any one of memory locations.

- The remaining bits will get either signed or unsigned extended bits depending upon the opcode and size which is selected by mux.

Overview of LD and ST memory interface switch combinations

MEMORY

REGISTER (32b)

ST32

ST16

ST8

LD32

LDU16

LDS16

LDU8

LDS8

D31

D0-31

D0-15, D8-23, D16-31

(SSSSSSSSSSSSSSSSSSSSSSSS), D8-15, D16-23, D16-31

(0000000000000000), D8-15, D16-23, D16-31

(SSSSSSSSSSSSSSSS), D8-15, D16-23, D16-31

(000000000000000000000000), D8-15, D16-23, D16-31

D0

D0-31

D0-7, D8-15, D16-23, D16-31

HAZARDS

TYPES OF HAZARDS

- Structural Hazards

- Control Hazards

- Data Hazards

STRUCTURAL HAZARDS

It is caused due to the fault in the structure/architecture of microprocessor. This is because pipeline stages(IF, FO, WB) will be communicating with only one memory.

Later this is resolved using STALL logic.

CONTROL HAZARDS

This is caused due to change in the normal operation of  PC like BRA,BRA COND, RETI, CALL, GOTO, Write PC instructions.

This can be resolved by FLUSH logic in the pipelines.

DATA HAZARDS

This caused to due data dependencies like Read After Write(RAW),Write After Read(WAR), Write After Write(WAW) conditions in instructions.

This can be resolved by data forwarding.

DATA HAZARDS

Consider an example,

MOV R0, Z

MOV R1, R0

In first instruction, R0 is being written. In second instruction, R0 is being read. But, when R0 is read, it is not yet been updated by new value which will be written in first instruction. Hence, wrong (old value) value of R0 will be read. So, 2nd instruction is dependent on 1st instruction to get new value (correct value). This is known as DATA DEPENDENCY. This condition is called Read After Write (RAW) Hazard.

RAW Hazard Detection

If ((RD_REGX_NUM|RR==WR_REGX_NUM|EX) && (RD_REGX_EN|RR=1 && WR_REGX_EN|EX=1))

Where Xϵ [A,B,C]

If the above condition is true, RAW is detected.

This can be resolved by delaying reading R0 by 2 methods

Method 1: Stall

In this method, we delay by adding bubble (STALL) to 2nd instruction. A bubble does not increment PC by 4. This is added prior to RR stage so that reading R0 gets delayed and by this time R0 gets updated with new value.

Instruction 1: F1	RR1	EX1	WB1

STALL

Instruction 2:		F2		RR2	EX2	WB2

Method 2: Adding NOP

Adding NOP increments PC by 4 and delays reading R0.

Instruction 1: F1	RR1	EX1	WB1

Instruction N: 		NOP	NOP	NOP	NOP

Instruction 2:			F2	RR2	EX2	WB2

By using the above 2 methods, we are wasting clock cycles unnecessarily and also in method 2, it makes the program bigger and busier. Hence, we use DATA FORWARDING as a solution.

DATA FORWARDING

Once the hazard is detected in RR stage, ALU knows about the hazard and then R0 is routed to that data from ALU and forward R0 directly to RR2 in the same clock telling RR2 not to read from register. Thus, ALU result i.e., new value is directly forwarded to R0 in RR2.

In this way, no clocks are wasted.

Instruction 1: F1	RR1	EX1	WB1

DATA FORWARDING

Instruction 2:		F2	RR2	EX2	WB2

The data can be forwarded from ALU, WRITE_REG_DATA(memory read), PC, PCT, SP, INC/DEC REG, FLAGS.

Case1: From ALU

If ((RD_REGA_NUM|RR==WR_REGA_NUM|EX) && (RD_REGA_EN|RR=1 && WR_REGA_EN|EX=1))  then WR_REGA_DATA=RD_REGA_DATA

If ((RD_REGB_NUM|RR==WR_REGB_NUM|EX) && (RD_REGB_EN|RR=1 && WR_REGB_EN|EX=1))  then WR_REGB_DATA=RD_REGB_DATA

If ((RD_REGC_NUM|RR==WR_REGC_NUM|EX) && (RD_REGC_EN|RR=1 && WR_REGC_EN|EX=1))  then WR_REGC_DATA=RD_REGC_DATA

Case2: WRITE_REG_DATA(memory read)

If ((RD_REGA_NUM|RR==WR_REGA_NUM|EX) && (RD_REGA_EN|RR=1 && WR_REGA_EN|EX=1))  then WR_REGA_DATA=WR_MEM_DATA

If ((RD_REGB_NUM|RR==WR_REGB_NUM|EX) && (RD_REGB_EN|RR=1 && WR_REGB_EN|EX=1))  then WR_REGB_DATA=WR_MEM_DATA

If ((RD_REGC_NUM|RR==WR_REGC_NUM|EX) && (RD_REGC_EN|RR=1 && WR_REGC_EN|EX=1))  then WR_REGC_DATA=WR_MEM_DATA

Case 3: PC

If ((RD_REGA_NUM|RR==WR_REGA_NUM|EX) && (RD_REGA_EN|RR=1 && WR_PC_EN|EX=1))  then WR_REGA_DATA=WR_PC_DATA

If ((RD_REGB_NUM|RR==WR_REGB_NUM|EX) && (RD_REGB_EN|RR=1 && WR_PC_EN|EX=1))  then WR_REGB_DATA=WR_PC_DATA

Case 4: PCT

If ((RD_REGA_NUM|RR==WR_REGA_NUM|EX) && (RD_REGA_EN|RR=1 && WR_PCT_EN|EX=1))  then WR_REGA_DATA=WR_PCT_DATA

If ((RD_REGB_NUM|RR==WR_REGB_NUM|EX) && (RD_REGB_EN|RR=1 && WR_PCT_EN|EX=1))  then WR_REGB_DATA=WR_PCT_DATA

Case 5: SP

If ((RD_REGA_NUM|RR==WR_REGA_NUM|EX) && (RD_REGA_EN|RR=1 && WR_SP_EN|EX=1))  then WR_REGA_DATA=WR_SP_DATA

If ((RD_REGB_NUM|RR==WR_REGB_NUM|EX) && (RD_REGB_EN|RR=1 && WR_SP_EN|EX=1))  then WR_REGB_DATA=WR_SP_DATA

Case 6: INC_DEC_REG

If ((RD_REGC_NUM|RR==WR_REGC_NUM|EX) && (RD_REGC_EN|RR=1 && WR_INC_DEC_EN|EX=1))  then WR_REGC_DATA=WR_INC_DEC_DATA

Case 7: FLAGS

If ((RD_REGA_NUM|RR==WR_REGA_NUM|EX) && (RD_REGA_EN|RR=1 && WR_FLAGS_EN|EX=1))  then WR_REGA_DATA=WR_FLAGS_DATA

If ((RD_REGB_NUM|RR==WR_REGB_NUM|EX) && (RD_REGB_EN|RR=1 && WR_FLAGS_EN|EX=1))  then WR_REGB_DATA=WR_FLAGS_DATA

STRUCTURAL HAZARD

To resolve this hazard, we use STALL logic.

STALL

It is used in between the pipeline stages. A stage can be STALLED due to Self-Stall (when it is not Done) or when next stage or previous stages are STALLED.

RR stage cannot Self Stall.

CLK

STALLN+1

EN

Q

D

STAGE N+1

STAGE N

INSERT_BUBBLE= STALLN v FLUSH

CASE1: When STAGE N is STALLED and STAGE N+1 is NOT STALLED

When STAGE N is STALLED, it inserts a bubble by making STALLN=1. No signals are sent to STAGE N+1 as STAGE N is not Done.

CASE2: When STAGE N+1 is STALLED and STAGE N is NOT STALLED

When STAGE N+1 is STALLED, it sends STALLN+1=1 making EN signal 0. Thus turning off DQ flip flop and thus prevents taking the data from previous stage as STAGE N+1 is not Done. Since DQ flip flop is disabled the inputs to it will be don’t care state.

CASE3: When both the stages are STALLED

When both the Stages are STALLED, the DQ flip flop will be disabled by STAGE N+1 and STAGE N will insert bubble. Since DQ flip flop is disabled the inputs(bubble) to it will be don’t care state.

TRUTH TABLE

| STALLN | STALLN+1 | EN | BUBBLE |
| --- | --- | --- | --- |
| 0 | 0 | 1 | 0 |
| 0 | 1 | 0 | X |
| 1 | 0 | 1 | 1 |
| 1 | 1 | 0 | X |

EN=STALLN+1

BUBBLE=STALLN

STALL=DONE=NOT_DONE

CONTROL HAZARD

This can be resolved by FLUSH logic.

FLUSH

Memory Interface

In this, bubble is sent to all stages and the value that was calculated in those stages are flushed.

Register Interface

DFF

DFF

DFF

Execute/ Fetch Operands

Read Register/ Address Generation

Instruction Fetch

Write Back

Flush

Stall

Bubble

Bubble

Bubble

When we write to PC, WB stage sends a FLUSH signal to all the stage by inserting bubble and all the old values are FLUSHED. Then new PC will be fetched by IF stage and normal operation continues.

PC Fetch will also be FLUSHED.

(WR_PC_EN|WB) v (WR_REG_EN|WB ^ WR_REG_NUM|WR=PC)              FLUSH=1

EXTERNAL INTERRUPT HANDLING

When interrupt instruction (INT ) is fetched by IF stage, we have to make sure that it is not flushed when PC is changed. So it has to be not Flushable.

If ((OPCODE=INT ) ^ ((WR_PC_EN|WB) v (WR_REG_EN|WB ^ WR_REG_NUM|WR=PC)

DO_NOT_FLUSH=1

Here, when any pipeline gets FLUSH and it has INT instruction, DO_NOT_FLUSH is made 1 i.e., not Flushable. Thus, it is not lost.